MOTIVA PAMC Sulfur Tank Lowered Emissions and Sulfur Pit Degassing

| Team Members - Ronald Pitman - Chris Ronquille - Daniel Benjamin - Anthony Musetti - Brandon Haldeman - Colby Miller - Ryan Vilce - Thomas Chapman - Todd Ellison - Doug Husen - Justin East | Special Thanks - ASRL - Rob Marriott - Elmo Nasato - Chuck Stephenson - Kenneth McIntush - Mike Anderson - Marco Van Son - Mukesh Mehta |
| --- | --- |

Port Arthur Manufacturing Complex (PAMC) Sulfur Tank

Abstract

By capturing learnings from recently published ASRL research data, a cross-functional Motiva team successfully reduced excess emissions from the SBU2 Sulfur Tank, which removed a regulatory agreement to construct a new replacement tank. This eliminated $30MM of low-return capital spend required by mid-2023 and progressed Motiva farther on its ESG journey.

The SBU2 Sulfur Tank (TK1928) was constructed and placed in service as part of the Motiva Crude Expansion Project (CEP) in 2012.  Since initial startup, sulfur tank emissions from TK1928 have caused an on-going Environmental and Industrial Hygiene threat to the site.

Due to the initial design and initial flawed engineering assumptions of “no in-pit H2S degassing but instantaneous in-tank H2S degassing”, the EXCESSIVE air sweep requirement of 31 MSCFH caused the Sulfur, Sulfur Dioxide (SO2) and some Hydrogen Sulfide (H2S) emissions to routinely exceed permitted authorized levels and restricted access to the tank due to personnel exposure risks. Over the past 9 years, the site has spent a tremendous amount of effort working numerous projects (scrubber design, scrubber redesign, installation of tank vents, attempting to re-permit emissions, etc.) to mitigate this threat. Ultimately, these attempts to mitigate excess emissions failed and Motiva reached an agreement with the TCEQ (Texas Environmental Regulatory Agency) to construct and operate a new replacement sulfur tank that would meet regulatory emission limits.  The estimated TIC for the project was $30MM and the tank was required to be in service (and TK1928 out of service) by July 1, 2023, per the agency agreement.

As the Capital Projects Team was progressing engineering design for this project in preparation to begin field construction, the Crude and Sulfur Business Team in partnership with Motiva Technical Excellence utilized new ASRL research and launched a final attempt to mitigate excess tank emissions.

A multi-disciplinary team, working with a high sense of urgency against a tight project start of construction schedule, evaluated ASRL data and implemented several operational changes that ultimately led to a regulatory tank stack test being performed in October 2021 that demonstrated SO2 and H2S emissions below the existing permit limits. Key operational changes implemented by the team:

- SRE / Brimstone FTIR H2S/H2Sx molten sulfur testing and Montrose vent stack testing were utilized on multiple test runs to verify safe operation at all times.

- Catalytic degassing at the sulfur pits was employed as a new technology to reduce H2S emissions.

- The team tenaciously pursued incrementally lowering the air sweep rate to the tank, which involved a detailed process safety hazard review, by restricting fresh air vents to reduce the formation of SO2 in the tank. Industry research information was published in early 2021 that identified excess tank air ingress as primary cause of SO2 formation.

- Sulfur pit sparging was transitioned from the use of air to nitrogen to further reduce the formation of SO2.

- Sulfur Tank Bubbler Level Instrument air flow rate was optimized with plans to change to Radar to further reduce the formation of SO2.

This initiative leveraged team members from multiple Motiva disciplines (Production, Process Engineering, Environmental, Process Safety, and Technical Excellence) to implement changes and support engineering testing for various operating scenarios on 15 different occasions over a six-month period.

These combined revisions have resulted in lowering the tank’s H2S and SO2 emissions by 69% (0.2 TPY) and 95% (9.2 TPY), respectively, which embodies Doing the Right Thing through sustainable operations and minimizing our footprint within the community.

Given that the emissions are now well within permitted limits as demonstrated by the regulatory stack test, Motiva worked with TCEQ to remove the agreement requiring construction of a new replacement tank, which eliminated $30MM in unnecessary capital spend.

The key enabler to this success was the transition to being wholly owned by Aramco. This transition facilitated access to 30-40 years’ worth of research data from the Alberta Sulphur Research Laboratory (ASRL) at University of Calgary in addition to participation in the Brimstone conference where SO2 emissions have been discussed numerous times.

With assistance from our industry research and contacts at Brimstone including ASRL, Elmo Nasato, Chuck Stephenson, Mike Anderson, Kenneth McIntush, Marco Van Son, and Mukesh Mehta, the MOTIVA team implemented liquid catalytic degassing to maintain safe operations while also minimizing SO2 emissions.  With the full support and alignment of the Business Team, SLT, and ELT, the team worked with a high sense of urgency embodying One Motiva and utilized these learnings to drive resolution to this long-standing threat delivering a huge win for Motiva and our community.

Background

The Port Arthur Area has a grandfathered small company operating a calciner without a Scrubber and is a major SO2 emitter contributing 10X’s the neighboring facilities combined in the area including Port Arthur and Beaumont.  This requires any new projects to be emissions neutral.  The Original Design basis also assumed zero In-pit H2S Degassing but Instantaneous H2S Degassing in the Sulfur Tank.  The sulfur transfer from the pits to the tank were designed to be performed in batch vs. continuous as well.  The emissions neutrality requirement and the instantaneous in-tank H2S degassing design basis required the Sulfur Tank to be constructed with redundant Scrubbers with 31,260 SCFH of Air Sweep.  The air sweep volumetric requirement for the design basis created excessive sulfur and SO2 emissions from the Sulfur Tank that had to be removed utilizing the Caustic Scrubber.

|  | Original Design | Comments |
| --- | --- | --- |
| Sulfur Tank Feed Rate | 3 Pits Pumping Max 206 gpm * 3 = 618 gpm = 5,931 LTPD | Assumes instantaneous maximum Sulfur Rate |
| H2S in Sulfur | 57 ppm 1 of 3 Degassers OOS | Average of 10 ppm, 10 ppm, 150 ppm H2S |
| H2S Evolution | 50% H2S instantaneous degasses | Assumes Instantaneous Degassing |
| Sweep Air Required for <25% of H2S LEL | 31,260 SCFH 750,240 SCFD | 25% LEL |

The dual Scrubbers were a failure operating 48-hours to a few weeks between outages.  Between 2012-2016, several efforts were performed to redesign the Scrubbers but were not successful including the following:

  - Original-designed packing replaced-in-kind twice

  - Installed newly designed:

    - Packing

    - Trays

    - Accumulator Internals

    - Demisters

    - Caustic filters and cooler

  - Installed VFD on blowers to regulate air flow

  - Capital Project

    - A $6.2 MM project began in 2013 (Shell Projects &Technology, Jacobs, and Motiva resources)

    - Proposed scope of work: two water wash columns for vapor leaving caustic scrubbers. revamp of caustic scrubber internals, new air blowers, water circulation pumps.  tank modifications to prevent excess air ingress

    - Project was canceled after it was determined that the proposed solution would be ineffective.

    - ~$3MM in regret spend

The redundant Scrubbers had multiple redesigns but were unsuccessful in processing the 31,260 SCFH or Sulfur Tank Air Sweep.  Below are some pictures of plugged packing, trays, and nozzles.

Sulfur Tank Vent changed from Scrubber to Center Stack

In 2016, to allow a safe Sulfur Tank operation without the concern for Scrubber pluggage and a potential >LEL atmosphere inside the Sulfur Tank, the TCEQ approved the Sulfur Tank to be vented through a vent stack until a new Sulfur Tank could be constructed with vapor recovery to the SRU’s.  With a review of the instantaneous degassing of the H2S from sulfur in the Tank, a center stack was installed and some of the air inlets were removed, which reduced the measured air sweep from 31,260 SCFH down to 19,000 SCFH.

The design change provided a path to maintain air sweep continuously for safety requirements; however, the SO2 and sulfur emissions required a future capital project for vapor recovery to an SRU.  The Sulfur Tank is located approximately 5,000 feet from the closest SRU.  Another issue was that the prevailing Gulf winds and number of air inlets created an environment where sulfur vapors exited the air inlets on the downwind side of the tank allowing SO2 to exit the tank at lower elevations.

The numerous (8) air inlets and prevailing Gulf winds created excessive air sweep and additional sulfur and SO2 emissions.  With the varying wind directions, signs of sulfur vapors could be seen exiting the center stack as well as varying air inlets.

Below is a closer picture of the vent stack and one of the eight jacketed air inlets.  Signs of sulfur vapor venting through the air inlets was evident.

Proposed New Sulfur Tank with Vapor Recovery with Location closer to SRU’s

The closest SRU (SRU5/6/7) to the Sulfur Tank (circled in green) is approximately 5,000 feet.  A vapor recovery project of this distance was considered; however, the risks and costs associated with vapor recovery line pluggage was prohibitive.  A New Sulfur Tank was being designed next to the SRU’s and installed with vapor recovery.

Steam Eductor Piping after 8-Years of Operation

With the Sulfur Tank ~5,000 feet from the nearest SRU and the proposed location of the New Sulfur Tank being a closer location, Vapor Recovery piping pluggage was a concern for long-term safe operation.  Plugged vapor recovery piping could lead to the loss of vapor space air sweep and a >LEL atmosphere.  Several sulfur tanks in industry have experienced deflagration due to plugged vapor recovery lines allowing >LEL to occur in the vapor space of sulfur tanks.

Below are pictures from June 2020 of one of the SRU’s constructed in 2012 with Sulfur Pit Vapor Recovery piping utilizing steam eductors.  The piping pluggage, corrosion, and operability issues of the associated XV valves have been problematic.  In the older SRU’s, Plant Air is used instead of steam for the eductors and have performed very well without pluggage issues.   Heated Plant Air was being considered for the Sulfur Tank Vapor Recovery but would require additional Plant Air Compressors.

Sulfur Tank Design Change using Research Data

Using research data and ASRL Chalk Talk research presented in early 2021, the design philosophy of zero in-pit H2S Degassing and Instantaneous H2S Degassing of Sulfur was changed to a consistent and conservative 24-hour H2S half-life in the sulfur pits and the sulfur tank.  The design assumption changed the air sweep requirement to be accomplished with tank breathing.  The PAMC sulfur is trucked and loads approximately 50-80 trucks per day.  This change significantly reduced the sulfur and SO2 emissions from the Sulfur Tank and moved any emissions to be from the center stack a higher elevation away from personnel.  As a conservative assumption, one of the sulfur pit degassing systems was out of service.  Further testing confirmed that the Sulfur Tank vapor space was always <1/4 LEL for H2S, which was consistent with the ASRL Chalk Talk data.

|  | Current Design | Comments |
| --- | --- | --- |
| Sulfur Tank Feed Rate | Environmental Permitted Maximum Sulfur Rate = 178 gpm = 1,710 LTPD | Realistic Sulfur Production throughout the day and capacity of the Sulfur Pit Product Chambers |
| H2S in Sulfur | 57 ppm 1 of 3 Degassers OOS | Average of 10 ppm, 10 ppm, 150 ppm H2S |
| H2S Evolution | 24-Hour half-life | Conservative 24-hour H2S half-life but realistically longer |
| Sweep Air Required for <25% of H2S LEL | Tank Breathing with truck loading | 25% LEL |

SRU Pit Air Sparge and H2S Testing Summary

Initial strategy included optimizing the Sulfur Pit Air Sparge Degassing to minimize SO2 entrainment to the Sulfur Tank.  Data with SRE testing utilizing the Brimstone FTIR analyzer confirmed the significantly lower sparge rates required for degassing <10 ppm Total H2S.

385 LTPD with 35-hour residence time (Design:  570 LTPD with 24-hour residence time)

4/28/2021-4/30/2021

-

Brimstone FTIR Analyzer

Developed by ASRL

SRU Pits Air Sparge, Morpholine, and H2S Testing Summary with SRE

SRU5/6/7 Sulfur Pits were placed under different operating conditions to document response times associated with air sparge and morpholine injection.  SRU5 Pit did not have Air Sparge throughout the testing and morpholine injection response after 6 hours.  Additional samples were conducted in the Lab under a vent hood that confirmed H2S removal rates using morpholine and agitation.  SRU6 Pit testing was conducted after Air Sparge was shut down for 20 hours to test rate of H2S increase once Air Sparge was discontinued.  SRU7 remained in normal operation with 32 MSCFH Air Sparge.  Additional confirmation was the H2S half-life in the sulfur.

SRU5 Sulfur Pit Testing:  385 LTPD, No Pit Degassing, and >35-hour residence time.

| SRU5 Pit 4-28-2021 | Time Tested | H2S (ppm) | H2SX (ppm) | Total H2S (ppm) |
| --- | --- | --- | --- | --- |
| SRU 5 Cooling Chamber | 07:45 | 171.2 | 114.4 | 285.6 |
| SRU 5 Degassing Chamber | 08:05 | 164.1 | 94.4 | 258.5 |
| SRU 5 Product Chamber | 08:12 | 138.0 | 85.3 | 223.3 |

SRU5 Sulfur Pit Testing:  385 LTPD, Morpholine injection at 14 ppm for 6 hours, No Pit Degassing, and >35-hour residence time.  Additional Morpholine testing performed in the Lab.

| SRU5 Pit 4-28-2021 | Time Tested | H2S (ppm) | H2SX (ppm) | Total H2S (ppm) |
| --- | --- | --- | --- | --- |
| SRU 5 Cooling Chamber | 13:02 | 186.9 | 33.1 | 220.0 |
| SRU 5 Degassing Chamber | 13:20 | 172.2 | 39.7 | 211.9 |
| SRU 5 Degassing Agitate with 2 drops (ml) Morph | 13:27 | 88.4 | 27.1 | 115.4 |
| SRU 5 Degassing Agitate with 2 drops (ml) Morph | 13:33 | 32.9 | 16.9 | 49.8 |
| SRU 5 Product Chamber | 13:40 | 159.8 | 26.5 | 186.3 |

SRU6 Sulfur Pit Testing:  385 LTPD, Pit Air Degassing stopped 4/27/2021 at 14:00 (2PM), and >35-hour residence time.  Testing showed response to Air Degassing shutdown for 20 hours.

| SRU6 Pit 4-28-2021 | Time Tested | H2S (ppm) | H2SX (ppm) | Total H2S (ppm) |
| --- | --- | --- | --- | --- |
| SRU 6 Cooling Chamber | 09:55 | 183.7 | 115.1 | 298.8 |
| SRU 6 Degassing Chamber | 10:10 | 151.2 | 31.4 | 182.6 |
| SRU 6 Product Chamber | 10:36 | 89.4 | 32.2 | 121.6 |

SRU7 Sulfur Pit Testing:  410 LTPD, 32 MSCFH Pit Air Degassing, and >24-hour residence time.

| SRU7 Pit 4-28-2021 | Time Tested | H2S (ppm) | H2SX (ppm) | Total H2S (ppm) |
| --- | --- | --- | --- | --- |
| SRU 7 Cooling Chamber | 08:40 | 165.3 | 115.8 | 281.1 |
| SRU 7 Degassing Chamber | 08:50 | 1.1 | 1.3 | 2.4 |
| SRU 7 Product Chamber | 08:55 | 2.1 | 1.7 | 3.8 |

Catalytic Degassing of Sulfur with Morpholine

With a target of 10-40 ppmw of morpholine, the H2S degassing to <10 ppm was achieved within our sulfur pits.  Data such as this produced from ASRL shows the H2S release rate as an example.  The three parameters that were ultimately changed through testing were morpholine injection into the sulfur rundown to catalytically release the H2Sx from the sulfur, air sparge in the sulfur pit changed to nitrogen sparge to reduce SO2 entrainment in the sulfur, reduction of air sweep at the sulfur tank, and optimization of air to bubbler level instrument at the sulfur tank.

Directional changes were observed between the Sulfur Pits and the Sulfur Tank are summarized in this table Directional Relations.

Sulfur Tank SO2 Emissions Reduction with Sulfur Degassing Testing Methods and associated Sulfur Tank Vapor Space Testing

Over a period of 6 months in 2021, methodological testing was performed to reduce the SO2 emissions from the Sulfur Tank while adhering to the <1/4 LEL (9,000 ppm H2S).  The original test data from 2018 was used as the basis.  Beginning with the June 9th testing, air sparge rates were reduced using the April 28th test data, morpholine injection targeting 40 ppm was initiated, and Tank Air Sweep was reduced by progressively removing the air inlets.  Beginning on July 22nd testing, air sparge was switched to nitrogen sparge in the Sulfur Pits.  During the August 24th testing, the final air inlet was removed from the Sulfur Tank and the bubbler level instrument was optimized.  Once the bubbler level instrument is changed to a radar level instrument, an evaluation will be performed to determine if the sulfur pit sparge can be switched back to air sparging vs. nitrogen sparging.

Objective was to always be less than 25% of LEL (<1/4 LEL) and below emissions limits.

8/17/2021 Breakout Table:  Sulfur Pits with Morpholine and N2 Sparge

8/24/2021 Breakout Table:

Sulfur Pits with Morpholine and N2 Sparge.  Sulfur Tank with Air Flow Reduction.

The August 24th testing with SRE using the Brimstone FTIR and Montrose performing the Sulfur Tank stack testing, varying operating conditions were performed to adhere to permitted emissions under all scenarios.  While verifying the vapor space <<1/4 LEL of H2S, the final air inlet was removed, and the Sulfur Tank was verified to only need air breathing without any appreciable increase in vapor space H2S concentrations.  The batch sulfur pit pumping rate was changed to only one pit pumping at a time and at a reduced pump out rate to the tank to minimize emissions at any given time.

Final Sulfur Tank Stack Testing conducted 10/13/2021

The final official state testing was conducted successfully in October 2021 with the results in the following table.  H2S concentrations were <<1/4 LEL.  SO2 concentration in the vapor space of ~559 ppm is closer to expected equilibrium from ASRL data, which will lower further to the ~400 ppm equilibrium concentration once the bubbler level instrument is changed to a radar level instrument.  Oxygen concentration is present to prevent FeS.  The average volumetric flow rate was 1,247 SCFH, which is much less than the original 31,260 SCFH.

Sulfur Tank Vent Stack during the State Testing with no visible Sulfur and SO2 plume.

Addendum from Research listed in References

SO2 is generated in both the molten sulfur and in the vapor space.  Minimizing air sparge rate, minimizing air sweep, and lowering sulfur pit and tank temperatures helped to reduce the overall SO2 emissions.  The following tables and graphs from the References were key research data to always maintain safe operations while utilizing reasonable reaction rates.

Mechanisms of SO2 formation in tanks storing degassed sulphur (1)

Appearance of SO2 over liquid sulphur as a function of time and 123°C to 180°C (1)

Sulfur Tank SO2 Concentrations with Excessive and Optimized Sulfur Pit Air Sparge Rates

Sulfur Tank SO2 with Excessive Sulfur Pit Air Sparge

SO2 concentration in the headspace of sulphur tanks storing degassed sulphur with excessive air-sparging.

Headspace gas analysis from liquid sulphur storage tank at sulphur plant (1)

Sulfur Tank SO2 with Optimized Sulfur Pit Air Sparge

SO2 concentration in the headspace of sulphur tanks storing degassed sulphur with optimized air-sparging.

Headspace gas analysis from liquid sulphur storage tanks at sulphur plant 2 (July 1999) (1)

Air Sparge Degassing

With air sparging, previous ASRL research provided the following correlation for H2S and SO2 in the vapor space.  (3)

H2S/SO2 Production During Air Sparge Degassing (3)

Morpholine Catalytic Degassing (3)

References

- UNDERSTANDING THE FORMATION OF AND HANDLING OF H2S AND SO2 EMISSIONS FROM LIQUID SULPHUR DURING STORAGE AND TRANSPORTATION – Sulphur Seminar 1999 –Jan A. Lagas, Stork Engineers & Contractors B.V.; P.D. Clark, G.D. Butlin and E. Fitzpatrick, Alberta Sulphur Research Ltd.; M.L.J.A. Wetzels and W.S. Kijlstra, Shell Global Solutions

- LIQUID SULPHUR DEGASSING: FUNDAMENTALS AND NEW TECHNOLOGY DEVELOPMENT IN SULPHUR RECOVERY - ASRL – Chalk Talk – 2012 – Peter D. Clark

- INVESTIGATIONS INTO THE CHEMICAL MECHANISMS OF LIQUID SULFUR DEGASSING AND THEIR RELEVANCE TO INDUSTRIAL DEGASSING SYSTEMS – ASRL, Quarterly Bulletin 1994 = Peter D. Clark, Kevin L. Lesage, Tracey McDonald, Anna Mason, and Aaron K. Neufeld